# Міністерство освіти і науки України Національний технічний університет України

“Київський політехнічний інститут імені Ігоря Сікорського” Факультет інформатики та обчислювальної техніки

# Кафедра інформаційних систем та технологій

# Лабораторна робота №5

Технології програмування на C/Embedded

«Communication interfaces (USART)»

# Виконав:

студент групи ІА-33

Мартинюк Ю.Р.

# Київ 2025

Тема: Communication interfaces (USART)

Мета: зрозуміти як працюють USART інтерфейс та як його налаштувати

Результат виконання роботи:

1) Посилання на додаток:

https://github.com/masonabor/C-Embedded/tree/main/lab5/lab5
2) Посилання на відео роботи:

https://github.com/masonabor/C-Embedded/blob/main/lab5/lab5.mp4

3) Трейс з логічного аналізатора:

https://github.com/masonabor/C-Embedded/blob/main/lab5/lab5.sal

Хід роботи:

Налаштування USART3

USART3 налаштовано на асинхронний режим, 19200 бод, 8 інформаційних бітів без перевірки на парність/непарність та з одним стоп бітом

Конфігурація виводів МК

Чотири виводи МК сконфігуровано на вихід (світлодіоди), PB10 та PB11 сконфігуровано як RX та TX для USART3

Код main.c

/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Main program body

******************************************************************************

* @attention

*

* Copyright (c) 2025 STMicroelectronics.

* All rights reserved.

*

* This software is licensed under terms that can be found in the LICENSE file

* in the root directory of this software component.

* If no LICENSE file comes with this software, it is provided AS-IS.

*

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

/* USER CODE END Includes */

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

/* USER CODE END PD */

/* Private macro -------------------------------------------------------------*/

/* USER CODE BEGIN PM */

/* USER CODE END PM */

/* Private variables ---------------------------------------------------------*/

UART_HandleTypeDef huart3;

/* USER CODE BEGIN PV */

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_USART3_UART_Init(void);

/* USER CODE BEGIN PFP */

void changeAllLed(GPIO_PinState state);

/* USER CODE END PFP */

/* Private user code ---------------------------------------------------------*/

/* USER CODE BEGIN 0 */

/* USER CODE END 0 */

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

/* MCU Configuration--------------------------------------------------------*/

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* USER CODE BEGIN Init */

/* USER CODE END Init */

/* Configure the system clock */

SystemClock_Config();

/* USER CODE BEGIN SysInit */

/* USER CODE END SysInit */

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_USART3_UART_Init();

/* USER CODE BEGIN 2 */

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1)

{

uint8_t rcvBuf[1];

HAL_StatusTypeDef result;

result = HAL_UART_Receive(&huart3, rcvBuf, 1, 10);

if (result == HAL_OK)

{

switch (rcvBuf[0])

{

case '1':

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_15, GPIO_PIN_SET);

HAL_UART_Transmit(&huart3, (uint8_t *)" - Blue ON\r\n", 12, 100);

break;

case '2':

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_15, GPIO_PIN_RESET);

HAL_UART_Transmit(&huart3, (uint8_t *)" - Blue OFF\r\n", 13, 100);

break;

case '3':

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_14, GPIO_PIN_SET);

HAL_UART_Transmit(&huart3, (uint8_t *)" - Red ON\r\n", 13, 100);

break;

case '4':

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_14, GPIO_PIN_RESET);

HAL_UART_Transmit(&huart3, (uint8_t *)" - Red OFF\r\n", 13, 100);

break;

case '5':

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_13, GPIO_PIN_SET);

HAL_UART_Transmit(&huart3, (uint8_t *)" - Orange ON\r\n", 13, 100);

break;

case '6':

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_13, GPIO_PIN_RESET);

HAL_UART_Transmit(&huart3, (uint8_t *)" - Orange OFF\r\n", 13, 100);

break;

case '7':

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_12, GPIO_PIN_SET);

HAL_UART_Transmit(&huart3, (uint8_t *)" - Green ON\r\n", 13, 100);

break;

case '8':

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_12, GPIO_PIN_RESET);

HAL_UART_Transmit(&huart3, (uint8_t *)" - Green OFF\r\n", 13, 100);

break;

case '9':

changeAllLed(GPIO_PIN_SET);

HAL_UART_Transmit(&huart3, (uint8_t *)" - All ON\r\n", 13, 100);

break;

case '0':

changeAllLed(GPIO_PIN_RESET);

HAL_UART_Transmit(&huart3, (uint8_t *)" - All OFF\r\n", 13, 100);

break;

default:

HAL_UART_Transmit(&huart3, (uint8_t *)" - UnexpCmd\r\n", 13, 100);

break;

}

}

}

/* USER CODE END 3 */

}

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Configure the main internal regulator output voltage

*/

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_NONE;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_HSI;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV1;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_0) != HAL_OK)

{

Error_Handler();

}

}

/**

* @brief USART3 Initialization Function

* @param None

* @retval None

*/

static void MX_USART3_UART_Init(void)

{

/* USER CODE BEGIN USART3_Init 0 */

/* USER CODE END USART3_Init 0 */

/* USER CODE BEGIN USART3_Init 1 */

/* USER CODE END USART3_Init 1 */

huart3.Instance = USART3;

huart3.Init.BaudRate = 19200;

huart3.Init.WordLength = UART_WORDLENGTH_8B;

huart3.Init.StopBits = UART_STOPBITS_1;

huart3.Init.Parity = UART_PARITY_NONE;

huart3.Init.Mode = UART_MODE_TX_RX;

huart3.Init.HwFlowCtl = UART_HWCONTROL_NONE;

huart3.Init.OverSampling = UART_OVERSAMPLING_16;

if (HAL_UART_Init(&huart3) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN USART3_Init 2 */

/* USER CODE END USART3_Init 2 */

}

/**

* @brief GPIO Initialization Function

* @param None

* @retval None

*/

static void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* USER CODE BEGIN MX_GPIO_Init_1 */

/* USER CODE END MX_GPIO_Init_1 */

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOB_CLK_ENABLE();

__HAL_RCC_GPIOD_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_12|GPIO_PIN_13|GPIO_PIN_14|GPIO_PIN_15, GPIO_PIN_RESET);

/*Configure GPIO pins : PD12 PD13 PD14 PD15 */

GPIO_InitStruct.Pin = GPIO_PIN_12|GPIO_PIN_13|GPIO_PIN_14|GPIO_PIN_15;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOD, &GPIO_InitStruct);

/* USER CODE BEGIN MX_GPIO_Init_2 */

/* USER CODE END MX_GPIO_Init_2 */

}

/* USER CODE BEGIN 4 */

void changeAllLed(GPIO_PinState state) {

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_12, state);

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_13, state);

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_14, state);

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_15, state);

}

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{

}

/* USER CODE END Error_Handler_Debug */

}

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */

Результат трейсу

На записі трейсу видно, як вмикаються та вимикаються світлодіоди в залежності від прийнятої команди по USART

Висновок: Під час виконання даної лабораторної роботи я ознайомився з принципом роботи інтерфейсу USART та на практиці навчився його використовувати для вмикання світлодіодів командами із клавіатури ПК.