# Міністерство освіти і науки України Національний технічний університет України

“Київський політехнічний інститут імені Ігоря Сікорського” Факультет інформатики та обчислювальної техніки

# Кафедра інформаційних систем та технологій

# Лабораторна робота №6

Технології програмування на C/Embedded

«Communication interfaces (I2C)»

# Виконав:

студент групи ІА-33

Мартинюк Ю.Р.

# Київ 2025

Тема: Communication interfaces (I2C)

Мета: зрозуміти як працюють I2C інтерфейс та як його налаштувати

Результат виконання роботи:

1) Посилання на додаток:

https://github.com/masonabor/C-Embedded/blob/main/lab6/lab6.zip
2) Посилання на відео роботи:

https://github.com/masonabor/C-Embedded/blob/main/lab6/lab6.mp4

3) Трейс з логічного аналізатора:

https://github.com/masonabor/C-Embedded/blob/main/lab6/lab6.sal

Хід роботи:

Налаштування I2C1

Налаштовуємо конфігурацію на частоту 100000, стандартний режим роботи та 7 бітну адресу

Конфігурація виводів МК

Конфігуруємо виводи I2C1, I2S3

Налаштування I2S3

Код main.c

/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Main program body

******************************************************************************

* @attention

*

* Copyright (c) 2025 STMicroelectronics.

* All rights reserved.

*

* This software is licensed under terms that can be found in the LICENSE file

* in the root directory of this software component.

* If no LICENSE file comes with this software, it is provided AS-IS.

*

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

/* USER CODE END Includes */

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

typedef enum {

NOTE_C4 = 0x00,

NOTE_C5 = 0x10,

NOTE_D5 = 0x20,

NOTE_E5 = 0x30,

NOTE_F5 = 0x40,

NOTE_G5 = 0x50,

NOTE_A5 = 0x60,

NOTE_B5 = 0x70,

NOTE_C6 = 0x80,

NOTE_D6 = 0x90,

NOTE_E6 = 0xA0,

NOTE_F6 = 0xB0,

NOTE_G6 = 0xC0,

NOTE_A6 = 0xD0,

NOTE_B6 = 0xE0,

NOTE_C7 = 0xF0,

NOTE_SILENCE = 0xFF

} soundToneType;

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

#define CS43L22_I2C_ADDRESS      0x94

#define I2C_TIMEOUT              10

/* USER CODE END PD */

/* Private macro -------------------------------------------------------------*/

/* USER CODE BEGIN PM */

/* USER CODE END PM */

/* Private variables ---------------------------------------------------------*/

I2C_HandleTypeDef hi2c1;

I2S_HandleTypeDef hi2s3;

DMA_HandleTypeDef hdma_spi3_tx;

/* USER CODE BEGIN PV */

int16_t data[100] = {0};

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_DMA_Init(void);

static void MX_I2C1_Init(void);

static void MX_I2S3_Init(void);

/* USER CODE BEGIN PFP */

/* USER CODE END PFP */

/* Private user code ---------------------------------------------------------*/

/* USER CODE BEGIN 0 */

void CS43L22_Init(void)

{

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_4, GPIO_PIN_SET);

uint8_t TxBuffer[2];

TxBuffer[0] = 0x0D;

TxBuffer[1] = 0x01;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x00;

TxBuffer[1] = 0x99;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x47;

TxBuffer[1] = 0x80;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x32;

TxBuffer[1] = 0xFF;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x32;

TxBuffer[1] = 0x7F;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x00;

TxBuffer[1] = 0x00;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x04;

TxBuffer[1] = 0xAF;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x0D;

TxBuffer[1] = 0x70;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x05;

TxBuffer[1] = 0x81;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x06;

TxBuffer[1] = 0x07;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x0A;

TxBuffer[1] = 0x00;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x27;

TxBuffer[1] = 0x00;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x1A;

TxBuffer[1] = 0x0A;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x1B;

TxBuffer[1] = 0x0A;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x1F;

TxBuffer[1] = 0x0F;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x02;

TxBuffer[1] = 0x9E;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*) &TxBuffer, 2, I2C_TIMEOUT);

}

void CS43L22_Beep(soundToneType pitch, uint32_t duration_ms)

{

uint8_t TxBuffer[2];

if (pitch == NOTE_SILENCE) {

HAL_Delay(duration_ms);

return;

}

TxBuffer[0] = 0x1D;

TxBuffer[1] = 0x00;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*)TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x1C;

TxBuffer[1] = (uint8_t)pitch;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*)TxBuffer, 2, I2C_TIMEOUT);

TxBuffer[0] = 0x1E;

TxBuffer[1] = 0xC0;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*)TxBuffer, 2, I2C_TIMEOUT);

HAL_Delay(duration_ms);

TxBuffer[0] = 0x1E;

TxBuffer[1] = 0x00;

HAL_I2C_Master_Transmit(&hi2c1, CS43L22_I2C_ADDRESS, (uint8_t*)TxBuffer, 2, I2C_TIMEOUT);

HAL_Delay(20);

}

void play(void)

{

const uint32_t tempo_multiplier = 1;

const uint32_t t_whole = 2000 / tempo_multiplier;

const uint32_t t_half  = 1000 / tempo_multiplier;

const uint32_t t_quart = 500  / tempo_multiplier;

typedef struct {

soundToneType note;

uint32_t duration;

} Note;

const Note melody[] = {

{NOTE_E6, t_quart},

{NOTE_A5, t_quart},

{NOTE_A5, t_quart},

{NOTE_B5, t_quart},

{NOTE_C6, t_quart},

{NOTE_B5, t_quart},

{NOTE_A5, t_quart},

{NOTE_B5, t_quart},

{NOTE_C6, t_quart},

{NOTE_B5, t_quart},

{NOTE_A5, t_quart},

{NOTE_E6, t_half},

{NOTE_SILENCE, t_quart},

{NOTE_E6, t_quart},

{NOTE_A5, t_quart},

{NOTE_A5, t_quart},

{NOTE_B5, t_quart},

{NOTE_C6, t_quart},

{NOTE_B5, t_quart},

{NOTE_A5, t_quart},

{NOTE_B5, t_quart},

{NOTE_C6, t_quart},

{NOTE_B5, t_quart},

{NOTE_A5, t_quart},

{NOTE_E6, t_half},

{NOTE_SILENCE, t_quart},

{NOTE_C6, t_quart},

{NOTE_C6, t_quart},

{NOTE_D6, t_quart},

{NOTE_E6, t_quart},

{NOTE_E6, t_quart},

{NOTE_D6, t_quart},

{NOTE_C6, t_quart},

{NOTE_D6, t_quart},

{NOTE_E6, t_quart},

{NOTE_D6, t_quart},

{NOTE_C6, t_quart},

{NOTE_B5, t_half},

{NOTE_SILENCE, t_quart},

{NOTE_A5, t_quart},

{NOTE_A5, t_quart},

{NOTE_B5, t_quart},

{NOTE_C6, t_quart},

{NOTE_B5, t_quart},

{NOTE_A5, t_quart},

{NOTE_B5, t_quart},

{NOTE_A5, t_half},

{NOTE_SILENCE, t_whole}

};

int totalNotes = sizeof(melody) / sizeof(melody[0]);

for (int i = 0; i < totalNotes; i++) {

CS43L22_Beep(melody[i].note, melody[i].duration);

}

}

/* USER CODE END 0 */

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

/* MCU Configuration--------------------------------------------------------*/

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* USER CODE BEGIN Init */

/* USER CODE END Init */

/* Configure the system clock */

SystemClock_Config();

/* USER CODE BEGIN SysInit */

/* USER CODE END SysInit */

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_DMA_Init();

MX_I2C1_Init();

MX_I2S3_Init();

CS43L22_Init();

HAL_I2S_Transmit_DMA(&hi2s3, (uint16_t *)data, 100);

/* USER CODE BEGIN 2 */

for (int i = 0; i < 3; i++) {

play();

}

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1)

{

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

}

/* USER CODE END 3 */

}

/**

* @brief System Clock Configuration

* @retval None

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

/** Macro to configure the PLL multiplication factor

*/

__HAL_RCC_PLL_PLLM_CONFIG(16);

/** Macro to configure the PLL clock source

*/

__HAL_RCC_PLL_PLLSOURCE_CONFIG(RCC_PLLSOURCE_HSI);

/** Configure the main internal regulator output voltage

*/

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

/** Initializes the RCC Oscillators according to the specified parameters

* in the RCC_OscInitTypeDef structure.

*/

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_NONE;

RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSI;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

/** Initializes the CPU, AHB and APB buses clocks

*/

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_HSI;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV1;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_0) != HAL_OK)

{

Error_Handler();

}

}

/**

* @brief I2C1 Initialization Function

* @param None

* @retval None

*/

static void MX_I2C1_Init(void)

{

/* USER CODE BEGIN I2C1_Init 0 */

/* USER CODE END I2C1_Init 0 */

/* USER CODE BEGIN I2C1_Init 1 */

/* USER CODE END I2C1_Init 1 */

hi2c1.Instance = I2C1;

hi2c1.Init.ClockSpeed = 100000;

hi2c1.Init.DutyCycle = I2C_DUTYCYCLE_2;

hi2c1.Init.OwnAddress1 = 0;

hi2c1.Init.AddressingMode = I2C_ADDRESSINGMODE_7BIT;

hi2c1.Init.DualAddressMode = I2C_DUALADDRESS_DISABLE;

hi2c1.Init.OwnAddress2 = 0;

hi2c1.Init.GeneralCallMode = I2C_GENERALCALL_DISABLE;

hi2c1.Init.NoStretchMode = I2C_NOSTRETCH_DISABLE;

if (HAL_I2C_Init(&hi2c1) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN I2C1_Init 2 */

/* USER CODE END I2C1_Init 2 */

}

/**

* @brief I2S3 Initialization Function

* @param None

* @retval None

*/

static void MX_I2S3_Init(void)

{

/* USER CODE BEGIN I2S3_Init 0 */

/* USER CODE END I2S3_Init 0 */

/* USER CODE BEGIN I2S3_Init 1 */

/* USER CODE END I2S3_Init 1 */

hi2s3.Instance = SPI3;

hi2s3.Init.Mode = I2S_MODE_MASTER_TX;

hi2s3.Init.Standard = I2S_STANDARD_PHILIPS;

hi2s3.Init.DataFormat = I2S_DATAFORMAT_16B;

hi2s3.Init.MCLKOutput = I2S_MCLKOUTPUT_ENABLE;

hi2s3.Init.AudioFreq = I2S_AUDIOFREQ_48K;

hi2s3.Init.CPOL = I2S_CPOL_LOW;

hi2s3.Init.ClockSource = I2S_CLOCK_PLL;

hi2s3.Init.FullDuplexMode = I2S_FULLDUPLEXMODE_DISABLE;

if (HAL_I2S_Init(&hi2s3) != HAL_OK)

{

Error_Handler();

}

/* USER CODE BEGIN I2S3_Init 2 */

/* USER CODE END I2S3_Init 2 */

}

/**

* Enable DMA controller clock

*/

static void MX_DMA_Init(void)

{

/* DMA controller clock enable */

__HAL_RCC_DMA1_CLK_ENABLE();

/* DMA interrupt init */

/* DMA1_Stream5_IRQn interrupt configuration */

HAL_NVIC_SetPriority(DMA1_Stream5_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(DMA1_Stream5_IRQn);

}

/**

* @brief GPIO Initialization Function

* @param None

* @retval None

*/

static void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* USER CODE BEGIN MX_GPIO_Init_1 */

/* USER CODE END MX_GPIO_Init_1 */

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOA_CLK_ENABLE();

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOD_CLK_ENABLE();

__HAL_RCC_GPIOB_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_4, GPIO_PIN_RESET);

/*Configure GPIO pin : PD4 */

GPIO_InitStruct.Pin = GPIO_PIN_4;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOD, &GPIO_InitStruct);

/* USER CODE BEGIN MX_GPIO_Init_2 */

/* USER CODE END MX_GPIO_Init_2 */

}

/* USER CODE BEGIN 4 */

/* USER CODE END 4 */

/**

* @brief  This function is executed in case of error occurrence.

* @retval None

*/

void Error_Handler(void)

{

/* USER CODE BEGIN Error_Handler_Debug */

/* User can add his own implementation to report the HAL error return state */

__disable_irq();

while (1)

{

}

/* USER CODE END Error_Handler_Debug */

}

#ifdef  USE_FULL_ASSERT

/**

* @brief  Reports the name of the source file and the source line number

*         where the assert_param error has occurred.

* @param  file: pointer to the source file name

* @param  line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t *file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif /* USE_FULL_ASSERT */

Висновок: Виконуючи дану роботу, я навчився програмувати обмін даними по шині I2C між STM32 та аудіо-кодеком CS43L22. Було досліджено карту регістрів кодека та реалізовано керування його вбудованим генератором частот. У результаті створено програму, яка відтворює задану мелодію шляхом послідовної передачі команд налаштування частоти та тривалості звучання. Отримані навички дозволяють реалізовувати просту звукову сигналізацію у вбудованих системах з мінімальним використанням обчислювальних ресурсів процесора.